# SZABIST Network – Master Config File

## 1. Global Device Security & Remote Management

Where to apply: Every single Router and Switch.

What it does: Encrypts all passwords and locks down the Console (physical cable), Telnet (remote access), and Privileged (`enable`) modes with the password `kkkr27`.

Crucial Addition/Update: Layer 2 switches require a Management IP and a Default Gateway to be accessed remotely across the network. Routers only need the password commands.

For Routers:

enable
configure terminal
service password-encryption
enable secret kkkr27
line console 0
password kkkr27
login
exit
line vty 0 4
password kkkr27
login
exit

For Switches (Includes the Management IP & Gateway):

enable
configure terminal
! 1. Tell the switch how to reply across the city
ip default-gateway [ROUTER_GATEWAY_IP]
! 2. Give the switch a Management IP Address
interface Vlan 1
ip address [UNUSED_IP_IN_SUBNET] [SUBNET_MASK]
no shutdown
exit
! 3. Apply the security passwords
service password-encryption
enable secret kkkr27
line console 0
password kkkr27
login
exit
line vty 0 4
password kkkr27
login
exit

## 2. Campus Switching (VLANs & Access Ports)

Where to apply: All Switches.

What it does: Creates the virtual departments (VLANs) and assigns specific PC ports to them. It also sets the connection between the switches and routers to "Trunk" mode so all VLANs can travel across one cable.

enable
configure terminal
! 1. Create the VLAN
vlan [VLAN_ID]
name [DEPARTMENT_NAME]
exit

! 2. Assign PC ports to the VLAN
interface range fastEthernet [START_PORT] - [END_PORT]
switchport mode access
switchport access vlan [VLAN_ID]
exit

! 3. Configure the Uplink (Cable going to another Switch or Router)
interface [GIGABIT_PORT]
switchport mode trunk
exit

## 3. Campus Routing (Inter-VLAN & DHCP Helper)

Where to apply: All Campus Routers (100, 99, 154, 153, 78, Villas).

What it does: Divides the router's single physical cable into virtual "sub-interfaces" so it can act as the Default Gateway for every VLAN. It also tells the router to forward IP address requests to your EDGE router.

enable
configure terminal
! Enter the virtual sub-interface for a specific VLAN
interface gigabitEthernet 0/0.[VLAN_ID]
encapsulation dot1Q [VLAN_ID]
ip address [GATEWAY_IP] [SUBNET_MASK]

! Forward DHCP requests to the EDGE Router
ip helper-address 11.11.11.1
exit

## 4. The Brains (OSPF Dynamic Routing)

Where to apply: All Routers (including EDGE).

What it does: Activates a mathematical algorithm that allows all routers to automatically share their connected networks with each other, forming a unified campus backbone.

enable
configure terminal
router ospf 1
! Advertise a network directly connected to this router
network [NETWORK_IP] [WILDCARD_MASK] area 0
exit

## 5. The Edge Router (DHCP & NAT to Internet)

Where to apply: The EDGE-RTR only.

What it does: Hosts the central DHCP server that hands out IPs to every campus. It also translates all internal private IPs into your public IP (11.11.11.1) so students can browse the internet.

enable
configure terminal
! 1. Create a DHCP Pool for a specific department
ip dhcp pool [POOL_NAME]
network [NETWORK_IP] [SUBNET_MASK]
default-router [GATEWAY_IP]
dns-server 8.8.8.8
exit

! 2. Setup NAT Translation (Inside to Outside)
interface [PORT_FACING_ISP]
ip nat outside
exit
interface [PORT_FACING_CAMPUS]
ip nat inside
exit

! 3. Activate NAT Overload
access-list 1 permit any
ip nat inside source list 1 interface [PORT_FACING_ISP] overload

! 4. Send all unknown traffic to the Internet (Default Route)
ip route 0.0.0.0 0.0.0.0 11.11.11.2

! 5. Broadcast the internet route to the other OSPF routers
router ospf 1
default-information originate
exit

## 6. Lab Security (Sticky MAC Port Security)

Where to apply: Sub-switches inside student labs (CS Lab, Mechatronics, etc.).

What it does: Memorizes the exact MAC address of the first PC plugged into the wall port. If a student unplugs the PC and plugs in their personal laptop, the port instantly drops the traffic.

Crucial Addition/Update: We utilized the `shutdown` violation instead of `restrict` to completely disable the compromised port, providing maximum physical security.

enable
configure terminal
interface range fastEthernet [START_PORT] - [END_PORT]
! Turn on Port Security
switchport port-security
! Limit to one device per port
switchport port-security maximum 1
! Automatically memorize the PC's MAC address
switchport port-security mac-address sticky
! Completely shut down the port if an unauthorized device connects
switchport port-security violation shutdown
exit

## 7. Network Firewalls (Access Control Lists - ACLs)

Where to apply: Main Campus Switches (via VLAN interfaces).

What it does: Blocks specific departments from talking to sensitive devices (like Faculty Printers or the ZABDESK server).

Crucial Addition/Update: Standard textbook logic dictates placing firewalls on the `out` direction of the destination VLAN. Because Packet Tracer simulates Layer 3 SVI firewalls poorly, we applied these lists in the `in` direction on all unauthorized incoming doors (VLANs and Uplinks).

enable
configure terminal
! 1. Create the Security Rule (Block List)
ip access-list extended [RULE_NAME]
deny ip any host [TARGET_DEVICE_IP]
permit ip any any
exit

! 2. Apply the rule to all UNAUTHORIZED incoming doors simultaneously
interface range vlan [UNAUTHORIZED_VLAN_1], vlan [UNAUTHORIZED_VLAN_2], [UPLINK_PORT]
ip access-group [RULE_NAME] in
exit

## 8. Permanent Storage (Saving Configurations)

Where to apply: Every single device.

What it does: Saves the current running configuration to the permanent NVRAM. Without this, all network settings will be erased upon a power cycle or software restart.

end
write memory

## 💡 Pro-Tips for your Presentation

Keep these short answers in mind if your instructor asks why you used certain technologies:

Why VLSM? "To prevent wasting IP addresses. We gave smaller masks like /28 to small departments and /26 to big labs."

Why OSPF? "Because if a cable breaks, OSPF automatically finds a backup path. Static routing requires manual fixing."

Why Port Security? "To prevent unauthorized devices from entering our network and causing IP conflicts or security breaches."

Why Inbound ACLs instead of Outbound? "We discovered a simulation bug in Packet Tracer's engine regarding outbound traffic on Switch Virtual Interfaces (SVIs). We engineered around this by catching malicious traffic inbound at the edge interfaces instead."

Why Default Gateways on Switches? "Layer 2 switches do not route traffic. To achieve the requirement of managing switches remotely from anywhere in the city, we had to give them a Default Gateway so they could send Telnet replies back across the routers."